Name & Student ID

Zhiguo Tao 20514075

Title

Plan for robot writer

Outline of the Problem to be Solved

Project Overview

This project aims to develop control software for a robotic writing system that converts text input into precise movement commands using G-code.

The program reads vector-based font data from SingleStrokeFont.txt, get the coordinates and pen movement data for each ASCII letter, and then convert them into G-code commands that the robot can execute.

Key Problem Components

1. Font Data Processing

- Read the SingleStrokeFont.txt file, which defines each character using single-stroke vector data.

- Save parsed data in memory structures for efficient access during drawing.

- Process the specific format: header line 999 C N followed by coordinate lines (X Y P) representing stroke points and pen state.

2. Text Input and Scaling

- Take user input for the desired text height, constrained to a range between 4 mm and 10 mm.

- Calculate scaling factor: scale = user_height / 18 (font's default height)

- Read text content from external files of arbitrary length

- Convert the content into ASCII letter and generate into G-code

3. Robot Communication

- Establish serial communication with Arduino at 115200

- Send G-code commands through virtual RS232 port

- Implement proper communication protocol: each command ends with \n, the program must wait for the Arduino to return an “ok” acknowledgment before sending the next command.

- Manage command buffering and synchronization to ensure smooth

Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| textContent | char* | Stores text content read from file |
| gcodeCommands | char* | Stores generated G-Code command sequence |
| drawingHeight | int | User-specified text height (4-10mm) |
| scaleFactor | float | Scaling factor calculated as: user_height / 18.0f |
| textLength | int | Length of text read from input file |
| charCount | int | Number of characters loaded from font file |

Function Declarations

int LoadFontData(const char* fontFilename, struct Character** fontArray, int* charCount)

Parameters:

fontFilename -- input, path to SingleStrokeFont.txt file

fontArray -- output, pointer to array of character definitions

charCount -- output, number of characters loaded

Return value: returns 1 if successful, 0 if failed

Int performTextDrawing(const char* textFilename, int textHeight)

Parameters:

textFilename -- Path to the text file to be drawn

textHeight -- Text height (4-10mm)

Return value: Returns 1 if the entire text file is processed successfully and all required G-code

commands are generated

Returns 0 if the text file cannot be opened, if the height value is invalid, or if an error occurs during G-code transmission

char* ReadTextFile(const char* filename, int* textLength)

Parameters:

filename -- input Path to the text file

textLength -- Output parameter, returns text length

Return value: Returns pointer to allocated text buffer, NULL if failed

char* GenerateDrawingGCode(const char* text, int textLength, int height, int* gcodeLength)

Parameters:

text -- Input text content

textLength -- input Text length

height -- input Text height

gcodeLength -- Output parameter, returns G-Code length

Return value: Returns pointer to generated G-Code buffer, NULL if failed

int SendToDrawingRobot(const char* gcode, int gcodeLength)

Parameters:

gcode -- G-Code command buffer

gcodeLength -- G-Code length

Return value: 1=all commands sent successfully, 0=communication failed

Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| LoadFontData | Normal font file | SingleStrokeFont.txt | Returns 1, successfully loads character data |
| LoadFontData | File not found | "missing_font.txt" | Returns 0, proper error handling |
| PerformTextDrawing | Normal drawing process | Valid text file, height=8 | Returns 1, all steps executed successfully |
| PerformTextDrawing | File not found | Invalid file path, height=6 | Returns 0, proper error handling |
| PerformTextDrawing | Invalid height | Height=2 | Returns 0, proper error handling |
| ReadTextFile | Normal text file | File containing "a quick fox" | Returns valid pointer, textLength=11 |
| ReadTextFile | Empty file | Empty text file | Returns NULL, textLength=0 |
| GenerateDrawingGCode | Single line text generation | Text="A", height=6 | Returns valid G-Code pointer with correct scaling commands |
| GenerateDrawingGCode | Multi-line text generation | Text="A\nB", height=8 | Returns valid G-Code pointer with line break handling |
| SendToDrawingRobot | Normal communication | Valid G-Code commands | Returns 1, robot responds normally |
| SendToDrawingRobot | Communication failure | Invalid G-Code or disconnected | Returns 0, proper error handling |
| main() | Complete workflow test | Valid (font + text files+height) | Program executes completely without errors |
| main() | Work error | inValid font or text files or height | Program stops and exit |

Flowchart(s)